Exercise:

Exercise 01:

Write following programs and fills bits of appropriate register and Carry Flag (CF)

Program 01:

Shift operand1 Left. The number of shifts is set by operand2. Algorithm:

Shift all bits left, the bit that goes off is set to CF.

Zero bit is inserted to the right-most position.

org 100h

MOV AL, 11100000b SHL AL, 1

RET

| AL |  |
| --- | --- |

| CF |  |
| --- | --- |

Program 02:

Shift operand1 Right. The number of shifts is set by operand2. Algorithm:

Shift all bits right, the bit that goes off is set to CF. Zero bit is inserted to the left-most position.

Org 100h

MOV AL, 00000111b SHR AL, 1

RET

| AL |  |
| --- | --- |
| CF |  |

Exercise 02:

In the following code sequence, show the value of AL after each shift or rotate instruction has executed.

mov al,0d4h

shr al,1 ;	AL=------

mov al,0d4h

sar al,1 ;	AL=------

mov al,0d4h

Department of Computer Sciences	51	Semester BS CS 03

sar al,4 ;	AL=------

mov al,0d4h

shl al,1 ;	AL=------

Exercise 03:

Which instruction shifts each bit in an operand to the left and copies the highest bit into both the Carry flag and the lowest bit position?

Exercise 04:

Write a program that 2-times shift left on following variables.

byte1 db 3Bh

byte2 db 46h

byte3 db 0FFh

Exercise 05:

Which instruction shifts each bit to the right, copies the lowest bit into the Carry flag, and copies the Carry flag into the highest bit position?

Exercise 06:

Write an assembly language program by using shift instruction that multiplies the operand by 2n. (n= no. of shifts)