COAL LAB # 3

Exercise # 1

- Largest 8-bit value will be obtained when all the bits are 1.(11111111)2

- Smallest 8-bit value is (00000000)2. However, we want smallest negative value so the MSB will be changed to 1, making it (10000000)2.

- Largest 16-bit value is (1111111111111111)2.

- Smallest 16-bit value is 0000000000000000. But we will change MSB to 1 to make it negative.

Exercise # 2:

- 10110011 (Binary to Hexadecimal)

- To convert binary to hexadecimal, we first separate the binary numbers into groups of four.

- Then, convert each four-bit binary number into Hexadecimal digit.

- After that, we combine all the obtained Hexadecimal digits into one number which will be the final result.

Manual Calculation:

Base Convertor:

- 455 (Decimal to Binary)

- To convert the Decimal to Binary, first find the largest power of 2 that fits in the decimal number.

- Subtract that power from the number.

- Repeat with the remaining value until you reach 0.

- Write 1s for the used powers and 0s for the missing ones.

Manual Calculation:

Base Calculator:

- 2AF (Hexadecimal to Binary)

- For converting Hexadecimal to Binary, each hexadecimal digit id converted to a four-bit binary number.

- Then all these four-bit binary numbers are combined to get the whole binary number.

Manual Calculation:

Base Calculator:

- 0A1 (Hexadecimal to Binary)

Manual Calculation:

Base Calculator:

Exercise # 3

- -1 = 11111111

- -17 = 11101111

- -19 = 11101101

- -62 = 11000010

- 127 = 01111111

- -128 = 10000000

Exercise # 4

- 01011100 = 92

- 11011100 = -36

- 10001111 = -113

- 01111110 = 126

- 10010001 = -111

- 10000000 = -128

Exercise # 5

- 7FB9 = 32697 (positive)

- 8123 = -32477 (negative)

- D000 = -12288 (negative)

- 6FFF = 28671 (positive)

Exercise # 6

Four software tools commonly used for assembly language programming are:

- Assembler (MASM (Microsoft Macro Assembler))

- Debugger (OllyDbg)

- Emulators (EMU8086, QEMU)

- Integrated Development Environment (MARS)